Structured flow of Dndsub1 for modem communications:

Start subroutine loop:

Do

I)Recycle Modem

A)Drop DTR (wait)

B)Raise DTR

C)Check Carrier

1)No carrier: exit do

2)Check count of tries: exit do if greater

Loop

Set computer to modem baud rate.

Do

Do

I)Send ATZ

A)Check OK returned: exit do if returned

1)Check number of tries: exit do if greater

Loop

Do

II)Send AT init

A)Check OK returned: exit do if returned

1)Check number of tries: exit do if greater

Loop

III)Check both OKs: exit do if returned

A)check count of tries: exit do if greater

Loop

Do

I)Check RING (or ring indicator bit)

A)Send ATA (If S0=0): or auto-answer (If S0>0)

II)Check Carrier bit: exit do if present

III)Check timer: goto top loop if expired

Loop

Do

I)Read modem response

II)Get 'Connect <baud>' modem to modem baud rate

(get special connect rates, such as: 'connect', or 'connect fast')

A)Compare against table

1)Check allowed baud: if not, display message, goto top loop

else, exit do

III)Check timer: exit do if expired

Loop

(do NOT change computer-to-modem baud rate to match modem-to-modem baud rate).

Enter MAIN..

Note: Standard modem bauds:

110, 150 (bios)

300, 600, 1200, 2400, 4800, 9600,

19200 (fossil replaces 110), 38400 (fossil replaces 150),

14400 (adjust to 9600),

28800 (adjust to 19200),

57600, 78600, 115200, 230400 (adjust to 38400),

Note: Nonstandard modem bauds:

450, 7200, 12000, 16800, 21600, 24000, 26400, 31200, 33600, 64000,

adjust as necessary.

-end-