How to use MavenCit with a FOSSIL driver:

(A quick hack at docs)

Well, basically you can just load say BNU or X00 and it'll work. But there's

a lot you can do to make it work BETTER (ok, two things)

1. You should set your buffers up better than it does. Minimal output

buffering is important, a big input buffer is real nice. MavenCit

uses a 4096 byte input buffer and a 256 byte output buffer internally.

The command to set this up under X00 is as follows:

DEVICE=X00.SYS R=4096 T=256

or for BNU:

BNU /R=4096 /T=256

... or even reduce the size of the output buffer further. 16 bytes is

PLENTY. Just drop it to the point where a user at your lowest baudrate

can hit 'S' and have it stop quickly enough to make them happy.

2. If you're using a high speed modem (HST, V.32, etc) lock your port rate

with the fossil for better throughput (14400 baud connects, etc.).

I lock mine at 38400 baud, and it works great. Things to watch out

for are:

Things which don't use a fossil driver will change the baudrate to

whatever the connect baud is if you don't tell them not to. Lots of

applications are ill-behaved this way. Either use my little utility

to change the output.apl thing (for things which use output.apl)

or hardcode your locked baud in a batch file (for things which take

command line parameters). WARNING: MOST APPLICATIONS USING OUTPUT.APL

AREN'T AGONNA LIKE RUNNING AT HIGH BAUDRATES!!! This includes (I think,

I'm not sure) things using The Dragon's CIT2DOOR program. A lot

of things like tradewars can handle a fossil, though.

Most protocols don't need to be told the speed and they should get the

right one. Be sure you don't specify a speed on the command line.

(i.e "DSZ port %p speed %s..." or suchlike) as it'll change the baudrate

(or it'll try, at least).

If you are using numeric response codes, you need to disable the

extra link information or switch to verbose response codes.

AT&A1 or AT&A0 works on US Robotics modems to tone down the numerics

to the proper levels. Ideally, you don't want it telling you what a

spiffy connection you got, just getting it.

Right. X00 command line to do this is:

DEVICE=X00.SYS B,0,38400

or for BNU:

BNU /L0,38400,8N1

(Note, that's for COM1, for COM2 use 1 in place of the 0 after B or L)

By The Way:

After testing with several versions of X00, I gave UP on it. It can't handle

output buffers under 512 bytes (quite a bit) and can't handle locked

databits/parity/stopbits, which it says it can do. It seems to work for

most people, but >I< for one am using BNU. It works for >ME<.

- The Maven

The Batchfile I use to start up MavenCit is as follows:

@ECHO OFF

BNU /M- /L1,38400,8N1 /R=4096 /T=32

CTDL %1 %2 %3 %4 %5 %6 %7 %8 %9

BNU /U

Which does the following:

1. Tells dos to not tell me about it.

2. Invokes BNU.COM without init messages (/M-), locking COM2 at 38400

baud, N81 (/L1,38400,8N1), and sets Recieve and Transmit Buffers

to 4kbytes and 32 bytes respectively.

3. Invokes MavenCit with whatever parameters I want to.

4. Unloads BNU (/U)

The /M- switch for BNU tells it to not pop up it's little "fossil status"

thing every time you init the port... like say, every time you shell out.

It gets annoying and mungs the screen if it displays them.

-----------------------------------------------------------------------------

High Speed Modem Notes:

1. You need to disable buffers internal to the modem on US Robotics modems

if you EVER want your users to be able to Pause or Stop. Add S15=8 to your

#MODSETUP string. This sets internal buffers to 128 bytes on normal

connections and 1.5k on ARQ/MNP connections (that's the best you can do)...

basically means you get 1-1/2 lines after you hit pause. Gads I wish USR

would just let you disable them entirely.

2. Set your FOSSIL buffers real, real small if you're using a locked

baudrate. Maybe 16 bytes, or even 8. (Transmit buffer, that is)

3. If MavenCit detects that the CTS (Clear to send) line is down, it

will pause output until either the line goes back up or a minute has passed.

If your CTS line is permanently down for some reason, don't use a

locked baud rate and set #CHECK_CTS in your config.cit to 0.

NOTE: Quite a few "minimal" serial cables do not have a CTS connection.

MAKE SURE YOURS DOES OR CIT WON'T WORK UNDER A FOSSIL.

4. Set MNP_WAIT to off. If you're running at 9600+, 99% chance you have

error correction (only exception I've ever seen is the "do it yourself

upgrade 2400->9600")

Here are my strings for a US Robotics Dual Standard....

#MODSETUP       "AT H0 V1 Q0 E0 M1 S0=1 S15=8 X4 &D2 &C1 &B1 &K1 &M4 &A1"

#DIAL_INIT      "AT H0 V1 S0=0 Q0 M1 X6 E1"

#DIAL_PREF      "ATDT"